___________________________________________________________

PC/TCP Packet Driver Specification

___________________________________________________________

Revision 1.05; document updated April 6, 1988

___________________________________________________________

FTP Software, Inc.

Cambridge, MA

___________________________________________________________

PC/TCP Packet Driver Specification

Note:  this document is public domain and may be distributed

freely and without  fee  or permission.  FTP Software's name

and  this notice must appear on  any	reproduction  of  this

document.

Support of  a hardware interface, or mention of an interface

manufacturer, by the Packet  Driver  specification  does not

necessarily indicate	that  the  manufacturer  endorses this

specification.

1  Introduction and Motivation

This document describes  the programming interface to PC/TCP

packet drivers.  Packet  drivers  provide  a	simple, common

programming  interface that allows multiple applications  to

share  a  network  interface	at  the data link level.   The

packet  drivers demultiplex  incoming  packets  amongst  the

applications by using the network media type field.

Different  versions  of  PC/TCP  exist for different network

media (Ethernet, 802.5 ring, serial lines), but  through the

use of the packet  driver,  the actual brand or model of the

network interface can be hidden from the application.

The packet driver provides calls  to	initiate  access  to a

specific packet type, to end access to it, to send a packet,

to  get  statistics  on  the	network interface and  to  get

information about the interface.

Protocol  implementations  that use the  packet  driver  can

completely coexist on a PC and can make use of one another's

services, whereas multiple applications which do not use the

driver do not coexist on  one machine properly.  Through use

of the packet driver, a user could run TCP/IP, Decnet, and a

proprietary  protocol  implementation   such	 as  Banyan's,

LifeNet's,  Novell's	or  3COM's  without  the  difficulties

associated with preempting the network interface.

Applications which use the packet driver can also run on new

network hardware of the same class  without  being modified;

only a new packet driver need be supplied.

Two  levels  of  packet  drivers  are   described   in  this

specification.   The first is the basic packet driver, which

provides minimal  functionality  but	should	be  simple  to

- 2 -

implement and which uses very few host resources.  The basic

driver provides operations to broadcast and receive packets.

The second driver is the  extended packet driver, which is a

superset of  the basic driver.  The extended driver supports

less commonly used functions of the  network	interface such

as  multicast, and also gathers statistics  on  use  of  the

interface and makes these available to the application.

Functions which are available in  only  the  extended packet

driver are noted as such in their descriptions.    All basic

packet  driver  functions  are  available  in  the  extended

driver.

2  Identifying network interfaces

Network interfaces  are  named  by  a  triplet  of integers,

<class, type, number>.  The first is the class of interface.

The class tells what	kind  of  media  the interface is for:

IEEE	802.3/Ethernet,  IEEE  802.5/Token  Ring,   ProNET-10,

Broadband Ethernet, Appletalk, serial line, etc.

The second number is the  type of interface:	this specifies

a particular	instance of an interface supporting a class of

medium.  Interface  types  for  Ethernet  might  name  these

interfaces:	 3COM  3C501   or   3C505,   Interlan  NI5010,

Univation,  BICC  Data  Networks ISOLAN, Ungermann-Bass NIC,

etc.	  Interface  types  for  IEEE 802.5 might  name  these

interfaces:  IBM Token Ring adapter, Proteon p1340, etc.

The last number is the interface number.  If	a  machine  is

equipped with more than one interface of a  class  and type,

the interfaces must be numbered to distinguish between them.

An appendix details constants  for  classes  and types.  The

class of an interface is an 8-bit integer, and its type is a

16 bit integer.  Class and type constants are managed by FTP

Software.    Contact	FTP  to  register a new class or  type

number.

The  type  0xFFFF  is  a  wildcard  type  which matches  any

interface  in the specified class.   It  is  unnecessary  to

wildcard interface numbers, as 0 will  always  correspond to

the first interface of the specified class and type.

This specification  has  no  provision  for  the  support of

multiple  network  interfaces  (with	similar  or  different

characteristics)  via a single Packet Driver and  associated

interrupt.  We feel  that  this  issue  is best addressed by

loading  several  Packet Drivers, one  per  interface,  with

different interrupts (although  all  may  be	included  in a

single  TSR  software  module).   Applications software must

check the class and type returned from an driver_info() call

already,  to make sure that the Packet  Driver  is  for  the

- 3 -

correct  media  and  packet  format.	  This	can easily  be

generalized by searching for another	Packet	Driver	if the

first is not of the right kind.

3  Initiating driver operations

The packet driver is invoked via a software interrupt in the

range 0x60 through 0x80.  This document does	not  specify a

particular interrupt, but describes a mechanism for locating

which  interrupt the driver uses.   The  interrupt  must  be

configurable	to  avoid   conflicts  with  other  pieces  of

software  that  also use software  interrupts.  The  program

which  installs  the	 packet  driver  should  provide  some

mechanism for the user to specify the interrupt.

The handler for the interrupt will start with a 3  byte jump

instruction, followed  by  the  null-terminated  text string

"PKT DRVR". To find the  interrupt being used by the driver,

an application should scan through the handlers  for vectors

0x60	through  0x80  until it finds one with the text string

"PKT DRVR" in it.

4  Programming interface

All  functions  are  accessed  via  the  software  interrupt

determined to be the driver's  via  the  mechanism described

earlier.  On entry,  register  AH  contains  the code of the

function desired.

The handle is an  arbitrary  integer	value  associated with

each type that  has  been  established  via  the access_type

call. Internally to the packet driver, it will probably be a

pointer, or a table offset.	 The  application  calling the

packet driver cannot depend on  it  assuming	any particular

range, or any other characteristics.

Note that some  of  the functions defined below are labelled

as  extended	driver	functions.    Because  these  are  not

required for basic network operations,  their implementation

may be considered  optional.	 Programs wishing to use these

functions  should use the driver_info function to  determine

if they are available in a given packet driver.

4.1  Entry conditions

FTP Software applications which call the  packet  driver are

coded in Microsoft  C  and assembly language.  All necessary

registers  are  saved  by  FTP's  routines  before  the  INT

instruction  to call the packet driver  is  executed.    Our

- 4 -

receiver() functions behave as  follows:    DS and the flags

are  saved  and  restored.	All  other  registers  may  be

modified,  and  should  be saved by the  packet  driver,  if

necessary.  Processor interrupts may be enabled while in the

routine  (only  by  older versions of PC/TCP).    On	entry,

receiver() switches to a local stack (re-entrant).

4.2  driver_info()

driver_info(handle)	  AH == 1 AL == FF

int handle;	  BX

error return:

carry flag set

error code		  DH

possible errors:

BAD_HANDLE

non-error return:

carry flag clear

version		  BX

class                   CH

type			  DX

number		  CL

name			  DS:SI

basic/extended	  AL

1 == basic, 2 == extended,

FF == not installed

This	function  returns  information	about	the  interface

associated with handle, which must  have  been  obtained via

access_type().

4.3  access_type()

int access_type(if_class, if_type, if_number, type,

typelen, receiver) AH == 2

int if_class;                   AL

int if_type;			  BX

int if_number;		  DL

char far *type;		  DS:SI

unsigned typelen;		  CX

int (far *receiver)();	  ES:DI

error return:

carry flag set

error code			  DH

possible errors:

BAD_HANDLE

NO_CLASS

NO_TYPE

NO_NUMBER

BAD_TYPE

- 5 -

NO_SPACE

TYPE_INUSE

non-error return:

carry flag clear

handle			  AX

receiver call:

(*receiver)(handle, flag, len [, buffer])

int handle;		  BX

int flag;		  AX

unsigned len;           CX

if AX == 1,

char far *buffer;	  DS:SI

Initiates  access to packets of the  specified  type.    The

argument type is  a  pointer to a packet type specification.

The  argument typelen is the length in  bytes  of  the  type

field.  The argument receiver is a pointer  to  a subroutine

which is called when a packet is received.

When a packet is received, receiver is called  twice	by the

packet driver.  The first time is to request a buffer   from

the  application  to	copy the packet into.  AX == 0 on this

call. The  application should return a pointer to the buffer

in ES:DI.  If the application has no buffers, it  may return

0:0  in  ES:DI,  and the driver should throw away the packet

and not perform the second call.

It is important that the packet length (CX) be valid	on the

AX == 0 call, so that the receiver can allocate a  buffer of

the proper size.  This length (as well as the copy performed

prior to the AX == 1 call) must include the  Ethernet header

and all received data, but not the trailing checksum.

On the second call, AX ==  1.   This call indicates that the

copy has been completed,    and the application may do as it

wishes  with the buffer.  The buffer	that  the  packet  was

copied into is pointed to by DS:SI.

4.4  release_type()

int release_type(handle)	  AH == 3

int handle;		  BX

error return:

carry flag set

error code			  DH

possible errors:

BAD_HANDLE

non-error return:

carry flag clear

- 6 -

This function  ends  access  to  packets  associated	with a

handle returned by  access_type().   The handle is no longer

valid.

4.5  send_pkt()

int send_pkt(buffer, length)	  AH == 4

char far *buffer;	  DS:SI

unsigned length;	  CX

error return:

carry flag set

error code			  DH

possible errors:

CANT_SEND

non-error return:

carry flag clear

Transmits length bytes of data,  starting  at  buffer.   The

application  must supply the entire packet, including  local

network  headers.  Any MAC or LLC  information  in  use  for

packet demultiplexing (e.g.  the  DEC-Intel-Xerox Ethertype)

must be filled in  by  the application as well.  This cannot

be performed by the  driver,	as no handle is specified in a

call to the send_packet function.

4.6  terminate()

terminate(handle)		  AH == 5

int handle;		  BX

error return:

carry flag set

error code			  DH

possible errors:

BAD_HANDLE

CANT_TERMINATE

non-error return:

carry flag clear

Terminates the driver associated with handle.   If possible,

the driver will exit and  allow MS-DOS to reclaim the memory

it was using.

4.7  get_address()

get_address(handle, buf, len)   AH == 6

int handle;		  BX

char far *buf;	  ES:DI

int len;		  CX

- 7 -

error return:

carry flag set

error code			  DH

possible errors:

BAD_HANDLE

NO_SPACE

non-error return:

carry flag clear

length		  CX

Copies the local net	address  associated  with  handle into

buf. The buffer buf is len bytes long.  The actual number of

bytes copied is returned in CX.  If the  NO_SPACE  error  is

returned, this indicates that len was  insufficient  to hold

the local net address.

4.8  reset_interface()

reset_interface(handle)	  AH == 7

int handle;		  BX

error return:

carry flag set

error code			  DH

possible errors:

BAD_HANDLE

non-error return:

carry flag clear

Resets the  interface  associated  with  handle  to  a known

state, aborting any  transmits in process and reinitializing

the receiver.  This call  has  been  included  primarily for

circumstances where a  high-level protocol has detected what

it thinks  may  be  an  interface  failure  or hang.	If the

packet driver  implementor has a high level of confidence in

the hardware, or the action  would  seriously  disrupt other

users of the interface, this can be treated as a no-op.

4.9  set_rcv_mode()

extended driver function

set_rcv_mode(handle, mode)	  AH == 20

int handle;		  BX

int mode;		  CX

error return:

carry flag set

error code		  DH

possible errors:

BAD_HANDLE

- 8 -

BAD_MODE

non-error return:

carry flag clear

Sets the  receive  mode  on  the  interface  associated with

handle. The following values are accepted for mode:

mode		meaning

1		turn off receiver

2		receive only packets sent to this interface

3		mode 2 plus broadcast packets

3		mode 3 plus limited multicast packets

4		mode 3 plus all multicast packets

5		all packets

Note that not all interfaces support all modes.  The receive

mode affects all packets  received  by  this	interface, not

just packets associated with	the  handle  argument.	As the

specification is currently written,  there  is  no provision

for  programming  "perfect  filters"	to   receive  specific

multicast addresses.

4.10	get_rcv_mode()

extended driver function

get_rcv_mode(handle, mode)	  AH == 21

int handle;		  BX

error return:

carry flag set

error code			  DH

possible errors:

BAD_HANDLE

non-error return:

carry flag clear

mode				  AX

Returns the current receive mode of the interface associated

with handle.

4.11	get_statistics()

extended driver function

get_statistics(handle)	  AH == 24

int handle;		  BX

error return:

carry flag set

error code			  DH

possible errors:

- 9 -

BAD_HANDLE

non-error return:

carry flag clear

char far *stats;		  DS:SI

statistics structure:

field                   byte length

packets in		  4

packets out		  4

bytes in		  4

bytes out		  4

errors in		  4

errors out		  4

packets dropped	  4

Returns a pointer to a statistics structure for this handle.

- 10 -

Appendix A

Interface classes and types

The following are defined as network interface classes, with

their  individual types  listed  immediately	following  the

class.

Ethernet/IEEE 802.3		     1

3COM 3C500/3C501         1

3COM 3C505		2

MICOM-Interlan NI5010	3

BICC Data Networks 4110	4

BICC Data Networks 4117	5

MICOM-Interlan NP600	6

Ungermann-Bass PC-NIC	8

Univation NC-516         9

TRW PC-2000		10

MICOM-Interlan NI5210	11

3COM 3C503		12

3COM 3C523		13

Western Digital WD8003	14

Spider Systems S4	15

ProNET-10			     2

Proteon p1300		1

IEEE 802.5/ProNET-4		     3

IBM Token ring adapter	1

Proteon p1340		2

Proteon p1344		3

Omninet			     4

Appletalk			     5

Serial line			     6

Starlan			     7

ArcNet			     8

Datapoint RIM		1

- 11 -

Appendix B

Function call numbers

The following numbers are used  to  specify  which operation

the packet driver should perform.  The number  is  stored in

register AH on call to the packet driver.

driver_info			1

access_type			2

release_type			3

send_pkt			4

terminate			5

get_address			6

reset_interface		7

*set_rcv_mode                 20

*get_rcv_mode                 21

*set_multicast_list		22

*get_multicast_list		23

*get_statistics		24

* indicates an extended packet driver function

- 12 -

Appendix C

Error codes

Packet driver calls indicate error by setting the carry flag

on return.   The  error  code  is returned in register DH (a

register not used to pass values to functions  must  be used

to  return  the  error code).  The following error codes are

defined:

1    BAD_HANDLE		invalid handle number

2    NO_CLASS                 no interfaces of specified class found

3    NO_TYPE			no interfaces of specified type found

4    NO_NUMBER		no interfaces of specified number found

5    BAD_TYPE                 bad packet type specified

6    NO_MULTICAST		this interface does not support

multicast

7    CANT_TERMINATE		this packet driver cannot terminate

8    BAD_MODE                 an invalid receiver mode was specified

9    NO_SPACE                 operation failed because of insufficient

space

10   TYPE_INUSE		the type had previously been accessed,

and not released.

11   BAD_COMMAND		the command was out of range, or not

implemented

12   CANT_SEND		the packet couldn't be sent (usually

hardware error)

- 13 -

Appendix D

802.3 vs. Blue Book Ethernet

One weakness of  the	present specification is that there is

no provision for simultaneous support of 802.3 and Blue Book

(the old DEC-Intel-Xerox standard)  Ethernet	headers.   The

problem is that the  Ethertype  of  Blue  Book packets is in

bytes   12   and   13  of  the  header,  and	in  802.3  the

corresponding bytes  are interpreted as a length.  In 802.3,

the field which would appear to be most useful to  begin the

type	check in is the 802.2 header,  starting  at  byte  14.

This	is  only  a  problem  on Ethernet and  variants  (e.g.

Starlan), where 802.3  headers  and  Blue  Book  headers are

likely to need co-exist for many years to come.

One solution is to redefine class 1 as  Blue	Book Ethernet,

and  define a parallel class for  802.3  with  802.2	packet

headers.  This requires that a 2nd Packet Driver (as defined

by its interrupt) be	implemented  where  it is necessary to

handle both kinds of	packets,  although  they could both be

part of the same TSR module.

I am holding class  9  for this purpose, but I am interested

in suggestions or comments from others.  I  would  also like

to hear comments on the  entry  conditions  for  the various

functions.

James B. VanBokkelen

FTP Software Inc.

P.O. Box 150

Kendall Sq. Branch P.O.

Boston, MA  02142

jbvb@vax.ftp.com

...!ftp!jbvb

- 14 -

Contents

1  Introduction and Motivation . . . . . . .  2

2  Identifying network interfaces  . . . . .  3

3  Initiating driver operations  . . . . . .  4

4  Programming interface . . . . . . . . . .  4

4.1  Entry conditions . . . . . . . . . .  4

4.2  driver_info()  . . . . . . . . . . .  5

4.3  access_type()  . . . . . . . . . . .  5

4.4  release_type() . . . . . . . . . . .  6

4.5  send_pkt() . . . . . . . . . . . . .  7

4.6  terminate()  . . . . . . . . . . . .  7

4.7  get_address()  . . . . . . . . . . .  7

4.8  reset_interface()  . . . . . . . . .  8

4.9  set_rcv_mode() . . . . . . . . . . .  8

4.10  get_rcv_mode()  . . . . . . . . . .  9

4.11  get_statistics()  . . . . . . . . .  9

Appendix A  Interface classes and types	       11

Appendix B  Function call numbers	       12

Appendix C  Error codes			       13

Appendix D  802.3 vs. Blue Book Ethernet        14

i