PCbridge: a inexpensive Ethernet Bridge

Vance Morrison

What is bridge?

A bridge is a device that spans two or more networks (almost always

ethernet), forwards packets between the networks so that it appears

that the networks are a single network.  Bridges differ from repeaters

because they gather information about where hosts on the networks are

and ONLY forward messages when necessary.  Since local traffic stays

local, a bridged network can handle more traffic than a single larger

network.  Bridges differ from routers (like PCroute), because they

only depend on the ethernet addressing scheme and not on the higher

level protocol (IP, DECNET, SNA etc), that handles internetwork routing.

What is PCbridge?

PCbridge is a simple program written for an IBM-PC that will turn

a PC with appropriate interface cards into a ethernet bridge.   It

has the capability to act as a standard 2 way ethernet-ethernet

or ethernet-starlan bridge as well as a remote bridge

(ethernet-serial-ethernet) or a n-way bridge.   With this software

it is possible to assemble a bridge for under $1000, that will

outperform bridges costing $2000 or more.

What does PCbridge Support?

PCbridge supports

Ethernet interfaces with the Western Digital WD8003E, WD8003EBT

WD8013EBT cards.

Starlan cards with the WD8003S and WD8003SH cards.

Serial Interfaces with the standard 8250 IBM COM ports at

baud rates up to 56.7K baud, as well as support for the

newer 16550AF chips with built in FIFO buffers (these

newer COM boards are HIGHLY recommended since they can

reduce the interrupt overhead by a factor of 10).  COM

boards with the 16550AF chips are now widely available

from PC vendors.

What does PCbridge NOT support?

Although it performs the  bridging function very well, it is not

a 'smart' bridge.  In particular it does NOT include any spanning

tree algorithm for detecting and breaking loops.    This does not

mean PCbridge can't be used in a bridged network with loops, it simply

means that other 'smarter' bridges must be used at strategic places

that will perform this function.  In addition PCbridge does NOT support

SNMP or any other network management protocol.

How fast is PCbridge?

Bridges have two metrics associated with them.  First is the number

of packets a second it can forward, and the other how many packets a second

it can filter.   The second metric is interesting because it can't be

easily be measured directly, after all rock connected to two pieces of

ethernet will filter (drop) as many packets per second as you like.  The

meaning of the second metric is to determine just how fast the router

can examine ethernet packets if it does not need to forward them.

To measure this I placed a counter in the main loop that was incremented

every time a packet was examined (from either ethernet segment).  Then I

modified the code slightly so that the code ALWAYS thought it got a packet

that it should drop.  Thus by running the code for a time and determining

how many packets it examined I could come up with a very accurate filtering

rate.

The forwarding rate was measured by measuring the packet delay.

This actually tends to underestimate the packet throughput, but we will

use it as a rough guide.    All of these tests were done with the

WD8003E card.  If you are using the WD8013 card with its 16 bit data

path, you can expect the packet forwarding rate to almost double.

Unfortunately, I do not have test data for this card.

Packet Filtering    Packet Forwarding (60byte)

rate (PPS)          rate (PPS)

-------------------------------------------------------------

4.77Mhz XT      4080 PPS            1677 PPS

10Mhz XT (est)  8000 PPS            3000 PPS

16Mhz AT        26214 PPS           5832 PPS

While packet filtering is independent of packet size, packet forwarding

is dependent on the size of the packet.   The packets I used where the

smallest ethernet packet (60 bytes).

For comparison purposes a low end Retix bridge costs $2000 (actually

more if you want thin ethernet) and has a forwarding rate of 6000 PPS

and a filtering rate of 10,000 PPS.   An 16Mhz AT with WD8013EBT cards

(costing about $1000) is likely to have forwarding rates of about

10,000 PPS and  filtering rates of about 25,000 PPS.  Thus you will

get almost twice the performance for half the cost.

------------------------------------------------------------------------

Which ethernet card is for me?

Note that all the WD80XX cards have on-board input buffers that can

keep up with the full 10Mbit/sec bit rate of ethernet.  The bridge

will ONLY start dropping packets when this input queue has filled.

The queue sizes are as follows

WD8003E         6.5K bytes

WD8003EBT       30.5K bytes

WD8013EBT       14.5K bytes

Note that may file serving programs (Novell and NFS for example) like

to send data in fast 8K blocks.  Because the WD8003E card has only a 6.5K

buffer, this will cause packets to be lost if the WD8003E card is used

in such an environment.

The WD8013EBT has the additional advantage of having a 16bit data path.

Thus this card can transfer data to other cards twice as fast as the

WD8003X cards.   The only disadvantage to this card is that it can not

be used in a XT machine, an AT or better machine is needed.

Thus the recommended configuration is a cheap AT clone with WD8013EBT

cards.  If an XT must be used, or packet forwarding is not a issue,

then WD8003EBT cards are the second choice.  Only if no file serving

traffic is expected (like in a remote bridge), is the WD8003E recommended.

Note that if you HAVE to run NFS though a bridge with WD8003E cards it

should be possible to ask NFS to send data in smaller 4K blocks.

On UNIX systems this is done via the 'rsize' and 'wsize' parameters

in fstab.   Since this has to be done for every NFS hosts, this is

clearly a choice of last resort, but I thought I would mention it.

--------------------------------------------------------------------------

Disclaimer -

I have tested PCbridge quite a bit, but we do not use it here

in a production environment.  Thus I cannot guarantee that it will

work well for your particular situation.  It should though.

---------------------------------------------------------------------------

Questions, comments, and bug reports to:

morrison@accuvax.nwu.edu

Vance Morrison